# INFORMATION SHEET No. 1.2

Resistors, Color coding, Conversion, Tolerance, Circuits and Ohm's Law

## Electronic Components

### DEVICES: RESISTORS

A resistor is an electronic device that limits or opposes the amount of current in a circuit. A resistor has two terminals across which electricity must pass, and is designed to drop the voltage of the current as it flows from one terminal to the next. A resistor is primarily used to create and maintain a known safe current within an electrical component.

Every resistor falls into one of two categories: fixed or variable.

1. Fixed resistor - has a predetermined amount of resistance to current

2. Variable resistor - can be adjusted to give different levels of resistance. Examples are the carbon composition and wirewound resistors. Variable resistors are also called potentiometers and are commonly used as volume controls on audio devices.

### Resistor Color Coding

Resistor - 4 Bands

| Band Color | Digit | Multiplier | Tolerance |
| --- | --- | --- | --- |
| Black | 0 | 1 |  |
| Brown | 1 | 10 | ±1% |
| Red | 2 | 100 | ±2% |
| Orange | 3 | 1,000 |  |
| Yellow | 4 | 10,000 | ±4% |
| Green | 5 | 100,000 |  |
| Blue | 6 | 1,000,000 |  |
| Violet | 7 | 10,000,000 |  |
| Grey | 8 | 100,000,000 |  |
| White | 9 |  |  |
| Gold |  | 0.1 | ±5% |
| Silver |  | 0.01 | ±10% |
| None |  |  | ±20% |

The color code chart is applicable to most of the common four-band and five-band resistors. Five-band resistors are usually precision resistors with tolerances of 1% and 2%. Most of the four-band resistors have tolerances of 5%, 10% and 20%.

The color codes of a resistor are read from left to right, with the tolerance band oriented to the right side. Match the color of the first band to its associated number under the digit column in the color chart. This is the first digit of the resistance value. Match the second band to its associated color under the digit column in the color chart to get the second digit of the resistance value.

Match the color band preceding the tolerance band (last band) to its associated number under the multiplier column on the chart. This number is the multiplier for the quantity previously indicated by the first two digits (four band resistor) or the first three digits (five band resistor) and is used to determine the total marked value of the resistor in ohms (see Ohm's Law, below).

### Tolerance

Is the maximum and minimum accepted value of resistor when measuring it thru multi-tester, by multiplying the percentage of the fourth or fifth colored value of the resistor on the first and second color, then after it adding and subtracting, adding it on the value of the resistor first and second color will get the maximum accepted value of the resistor and subtracting it will get the minimum.

brown, black, red, gold (5% tolerance) - Sample computation:

1000 - resistor's value

x 0.05 - resistor's 5% value

50 - conversion

1000 - resistor's value / 50 - resistor's 5% value

1050 - maximum tolerance

1000 - resistor's value / 50 - resistor's 5% value

950 - minimum tolerance

brown, black, red, silver (10% tolerance) - Sample computation:

1000 - resistor's value

x 0.1 - resistor's tolerance

100 - resistor's 10% value

1000 - resistor's value / 100 - resistor's 20% value

1100 - maximum tolerance

1000 - resistor's value / 100 - resistor's 10% value

900 - minimum tolerance

### How to test the Resistor using multi-tester

(Refer to diagrams showing multi-tester readings for different resistance values)

## Ohm's Law

Ohm's Law states that an electrical circuit's current is directly proportional to its voltage, and inversely proportional to its resistance. So, if voltage increases, for example, the current will also increase, and if resistance increases, current decreases; both situations directly influence the efficiency of electrical circuits. To understand Ohm's Law, it's important to understand the concepts of voltage, and resistance: current is the flow of an electric charge, voltage is the force that drives the current in one direction, and resistance is the opposition of an object to having current pass through it.

The formula for Ohm's Law is:

I = V/R       V = I x R       R = V/I

where:

V = voltage in volts

I = current in amperes

R = resistance in ohms

This formula can be used to analyze the voltage, current, and resistance of electricity circuits. Depending on what you are trying to solve we can rearrange it two other ways.

All of these variations of Ohm's Law are mathematically equal to one another. Let's look at what Ohm's Law tells us. In the first version of the formula, I = V/R, Ohm's Law tells us that the electrical current in a circuit can be calculated by dividing the voltage by the resistance. In other words, the current is directly proportional to the voltage and inversely proportional to the resistance. So, an increase in the voltage will increase the current as long as the resistance is held constant. Alternately, if the resistance in a circuit is increased and the voltage does not change, the current will decrease.

The second version of the formula tells us that the voltage can be calculated if the current and the resistance in a circuit are known. It can be seen from the equation that if either the current or the resistance is increased in the circuit (while the other is unchanged), the voltage will also have to increase.

The third version of the formula tells us that we can calculate the resistance in a circuit if the voltage and current are known. If the current is held constant, an increase in voltage will result in an increase in resistance. Alternately, an increase in current while holding the voltage constant will result in a decrease in resistance. It should be noted that Ohm's law holds true for semiconductors, but for a wide variety of materials (such as metals) the resistance is fixed and does not depend on the amount of current or the amount of voltage.

As you can see, voltage, current, and resistance are mathematically, as well as, physically related to each other. We cannot deal with electricity without all three of these properties being considered.

Voltage = 100 volts

Resistance = 100 ohms(Ω)

Current = ?

Answer: 1 ampere

Formula = I = V/R

(The symbol for an Ohm looks like a horseshoe and is pictured after the "100" in the diagram above.)

## ELECTRICAL CIRCUITS

An electrical circuit is a device that uses electricity to perform a task, such as run a vacuum or power a lamp. The circuit is a closed loop formed by a power source, wires, a fuse, a load, and a switch. Electricity flows through the circuit and is delivered to the object it is powering, such as the vacuum motor or light bulb, after which the electricity is sent back to the original source; this return of electricity enables the circuit to keep the electricity current flowing.

### Types of Circuits

A Series Circuit is the simplest because it has only one possible path that the electrical current may flow; if the electrical circuit is broken, none of the load devices will work.

In the Parallel Circuit, the electricity can travel to two or more paths. The path in which the electricity travels are separate and if ever one of its path fails to function, the electricity can still flow back to the source through the other paths.

A Series-Parallel Circuit, however, is a combination of the first two. It attaches some of the loads to a series circuit and others to parallel circuits.

Series Circuit Formula:

R_total = R₁ + R₂ + R₃

Parallel Circuit Formula:

R_total = 1 / (1/R₁ + 1/R₂ + 1/R₃)

Formula for 2 Parallel Circuit only:

R_T = (R1 x R2) / (R1 + R2)

### Series Circuits

Series Circuits are the simplest to work with. Here we have three resistors of different resistances. They share a single connection point. When added together the total resistance is 18kΩ.

### Calculating Total Resistance of a Parallel Circuit

R_T = 1 / (1/R1 + 1/R2 + 1/R3 + 1/R4 + 1/R5)

0.2 + 0.0714 + 0.0476 + 0.04 + 0.01 = 0.369

R_T = 1/0.369 = 2.7Ω

### Series-Parallel Circuits

Here we can use the shorter Product Over Sum equation as we only have two parallel resistors:

R_P1 = (R1 x R2) / (R1 + R2) = 37 x 24 / 37 + 24 = 416/61

R_P1 = 11.0491 + R3 + R₄

R_T = 11.049 + 58 = 75.0491

R_T = 7Ω

(Prepare for a Self check and Task Sheet, please provide a sheet of paper as answer sheet)

## Self Check No. 1.2.1

(Resistors, Color coding, Conversion, Tolerance, Circuits and Ohm's Law)

Instruction: choose the letter of the correct answer.

1. It is an electronic device that resists, limits or opposes the amount of current in a circuit:

a. resistor
   b. capacitor
   c. diode
   d. none of the above

2. A kind of resistor that can varies the resistance:

a. Fixed resistor
   b. Potentiometer
   c. Volume control
   d. Variable resistor

3. A circuit with only one possible path that the electrical current may flow:

a. Parallel
   b. Series
   c. Series - Parallel
   d. none of the above

4. the computation of accepted value of resistor's resistance from minimum to maximum:

a. decoding
   b. conversion
   c. tolerance
   d. none of the above

5. It states that an electrical circuit's current is directly proportional to its voltage, and inversely proportional to its resistance:

a. Fuse law
   b. ohm's law
   c. volt's law
   d. none of the above

## Self Check No. 1.2.2

(Resistors, Color coding, Conversion, Tolerance, Circuits and Ohm's Law)

Instruction: Write and illustrate the resistor's color coding table in exact order.

| COLOR | DIGIT | MULTIPLIER | TOLERANCE |
| --- | --- | --- | --- |

## Self Check No. 1.2.3

(Resistors, Color coding, Conversion, Tolerance, Circuits and Ohm's Law)

Resistor's Color Coding, Decoding and Conversion

Instruction: Give the given value of each color of resistor 1-5 and give the color of each given value of resistor 6-10.

1. Brown    Black    Red    Gold

2. Green    Blue    Yellow    Gold

3. Violet    Red    Orange    Silver

4. Yellow    White    Gold    Gold

5. Orange    Orange    Brown    Gold

6. 680 KΩ ± 5%

7. 26 KΩ ± 10%

8. 3.8 MΩ ± 5%

9. 4.7 KΩ ± 10%

10. 30 Ω ± 5%

## Self Check No. 1.2.4

(Resistors, Color coding, Conversion, Tolerance, Circuits and Ohm's Law)

Resistor's Tolerance

Instruction: Give all the details of each Resistor, identify the color and value, write down the minimum and maximum tolerance (1 pts. Each).

1. Red    Blue    Red    Gold    Silver

2. Gray    Orange    Yellow    Gold

3. White    Blue    Brown    Gold

4. Blue    Black    Gold    Gold

5. Violet    Red    Gold    Silver

6. 940 KΩ ± 5%

7. 77 KΩ ± 10%

8. 56 MΩ ± 5%

9. 4.7 KΩ ± 10%

10. 87 Ω ± 5%

## Homework No. 1.2.1

(Resistors, Color coding, Conversion, Tolerance, Circuits and Ohm's Law)

Resistor's Tolerance

Instruction: Give all the details of each Resistor, identify the color and value, write down the minimum and maximum tolerance (3 pts. Each).

1. Yellow    Violet    Yellow    Gold

2. Green    Blue    Brown    Gold

3. Red    White    Orange    Silver

4. Brown    Black    Red    Gold

5. Gray    Orange    Green    Silver

6. 10 KΩ ± 5%

7. 950 KΩ ± 10%

8. 8.2 MΩ ± 5%

9. 1.2 KΩ ± 10%

10. 100 Ω ± 5%

## Task Sheet No. 1.2.1

(Checking and Testing of Resistor)

Performance Objectives:

To understand the value of color coded resistor, to maximize the use of Multi-meter and to identify the conditions of a resistor.

Procedures:

1. Decode the color thru value

2. Compute the minimum and maximum tolerance

3. Using the suggested ohm's range, check using the multi-meter

4. Write down the parts, description (R1,R2,etc.), actual reading (resistance), the range use and the remarks if good or defective

5. Submit to Trainer for checking

| PARTS | DESCRIPTION | ACTUAL READING | RANGE | REMARKS |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

## Performance Criteria 1.2.1

(Checking and Testing of Resistor)

- Observe and follow safety policies and procedures?

- Use proper personal protective equipment?

- Did I identify different kind of electronic components?

- Follow measuring and correct ranging of Multi-meter?

- I used tools and equipments properly?

## Performance Criteria Checklist 1.2.1

(Checking and Testing of Resistor)

| RATING | REMARKS |
| --- | --- |
| 1 | Poor |
| 2 | Fair |
| 3 | Good |
| 4 | Satisfactory |
| 5 | Excellent |