# INFORMATION SHEET No. 1.3

Capacitors and Diodes

## DEVICES: CAPACITORS

A capacitor is an electronic device or components that store or accumulate electrical energy in a circuit. A capacitor is a tool consisting of two conductive plates, each of which hosts an opposite charge. These plates are separated by a dielectric or other form of insulator, which helps them maintain an electric charge.

Farad – is the measured unit of a capacitance, the higher the farad, the longer it holds the stored charges; the lower the farad, the shorter it holds the stored charges.

Capacitance is the quantity of electrical energy that a capacitor can store. Capacitance is dependent upon:

- Thickness of dielectric

- Area of the plates

- Constant of dielectric

The capacitor's functions are as follows:

Coupling – to prevent DC from entering the circuit.

De-coupling – to prevent or protect the circuit from another circuit.

Noisefilters or Snubbers – to protect the circuit from distortion and interference.

Motorstarter – is used for starting the motor.

Tuned-in Circuit – is used for tuning circuit.

### Types of Capacitors

- Polystyrene (Film capacitor)

- Mylar (for high voltage charge)

- Polyethylene (Film capacitor)

- Mica (for high voltage charge)

- Tantalum (electrolytic type)

- Ceramic (for small amount of charge)

Electrolytic capacitors have several prominent characteristics, including:

a. High capacitance-to-size ratio;

b. Polarity sensitivity and terminals marked + and -;

c. Allow more leakage current than other types; and

d. have their C value and voltage rating printed on them.

The main advantage of the electrolytic capacitor is the large capacitance-per-size factor. Two obvious disadvantages are the polarity, which must be observed, and the higher leakage current feature.

### Variable Capacitor

A variable capacitor is a special type of capacitor, most commonly used for tuning ratios, which allows the amount of electrical charge it can hold to be altered over a certain range, measured in a unit known as farads. Regular capacitors build up and store an electrical charge until it's ready to use. While a variable capacitor stores the charge in the same fashion, it can be adjusted as many times as desired to store different amounts of electricity.

Two types of variable capacitors include air variable capacitors and vacuum variable capacitors.

### How to test the capacitor using multi-tester?

Good Capacitor:

(Testing electrolytic capacitor: Ohmmeter connection - OHMS, Range - R x 1, Forward Resistance - Low, Capacitance - depends on value, Condition - Good)

Shorted Capacitor:

(Testing electrolytic capacitor: Range - R x 1, Forward Resistance - Zero, Condition - Shorted)

Open Capacitor:

(Testing electrolytic capacitor: Range - R x 10, Forward Resistance - Infinite, Capacitance - 470uF, Condition - Open)

Leaky Capacitor:

(Testing electrolytic capacitor: Range - R x 10, Forward Resistance - Zero, Capacitance - 100 uF, Condition - Leaky)

## DEVICES: DIODE

A diode is a semi-conductor device that permits the flow of current in only one direction. A Diode can convert electric current from AC to DC or from Alternating Current to Direct Current. This is called Rectification, and rectifier diodes are most commonly used in low current power supplies.

Rectification is a process of converting AC to DC.

A diode has 2 parts:

Anode (A) +       Cathode (K) -

### TYPES OF DIODE

1. Rectifier Diode - Converts AC to DC.

2. Signal Diode

Both diodes work the same way by allowing current to flow in one direction. The differences have to do with power and frequency characteristics. They are made from a p-n junction and are two lead devices.

Small signal diodes have much lower power and current ratings, around 150mA, 500mW maximum compared to rectifier diodes, they can also function better in high frequency applications or in clipping and switching applications with short-duration pulse waveforms.

- converts Intermediate Frequency (IF) to Audio Frequency (AF).

3. Regulator Diode - used for voltage regulation.

4. Temperature Dependent Diode - used for temperatures hot or cold to automatically on and off.

5. Light Emitting Diode (L.E.D)

is a semiconductor light source. LEDs are used as indicator lamps in many devices and are increasingly used for other lighting. Photodiode or Photo Sensitive Diode - Allows current flow when exposed to light (vice versa).

6. Photodiode (Photosensitive diode)

is a type of photodetector capable of converting light into either current or voltage, depending upon the mode of operation. The common, traditional solar cell used to generate electric solar power is a large area photodiode.

### How to test the Diode using multi-tester?

Good Diode:

(Ohmmeter connection: Range R x 10, Forward Resistance - LOW (Not zero/not infinite), Part No. - 1N4001, Reverse: Range R x 10, Reverse Resistance - Infinite, Condition - Good)

Shorted Diode:

(Range R x 1, Forward Resistance - Zero, Reverse: Range R x 1, Reverse Resistance - Zero, Condition - Shorted)

Open Diode:

(Range R x 10, Forward Resistance - LOW (Not zero/not infinite), Part No. - 1N2Rs, Reverse: Range R x 10, Reverse Resistance - Infinite/zero, Condition - Open)

Leaky Diode:

(Range R x 1, Forward Resistance - Zero, Reverse: Range R x 1, Reverse Resistance - Zero, Condition - Shorted)

(Prepare for a Self check and Task Sheet, please provide a sheet of paper as answer sheet)

## Self Check No. 1.3.1

(Capacitors and diodes)

Instruction: Enumeration

1-4. Give at least 4 kinds of diodes and illustrate the schematic symbol.

5-8. Give at least 4 kinds of capacitors.

9-10. 2 parts of Diode

Instruction: write down the condition of each measured components as leaky, shorted, open or good. (2 pts. Each)

(Refer to diagrams showing various multi-tester readings for diodes and capacitors)

## Task Sheet No. 1.3.1

(Checking and Testing of Capacitor)

Performance Objectives:

To understand the value of capacitor, to intensify the use of Multi-meter and to identify the conditions of a capacitor.

Procedures:

1. Check the value of capacitor

2. Check the polarity of the terminal

3. Using the suggested range in testing a capacitor

4. Use the tester in Reverse bias

5. Write down the parts, description (C1,C2,etc.), findings, the range use and the remarks if good or defective such as leaky, shorted or open.

6. Submit to Trainer for checking

| PARTS | DESCRIPTION | FINDINGS | RANGE | REMARKS |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

## Task Sheet No. 1.3.2

(Checking and Testing of Diode)

Performance Objectives:

To understand the value of diode, to intensify the use of Multi-meter and to identify the conditions of a diode.

Procedures:

1. Check the type of diode

2. Check the polarity of the terminal

3. Using the multi-meter, check by performing both reverse and forward bias

4. Write down the parts, description (D1,D2,etc.), findings, the range use and the remarks if good or defective such as leaky, shorted or open.

5. Submit to Trainer for checking

| PARTS | DESCRIPTION | FINDINGS | RANGE | REMARKS |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

## Performance Criteria 1.3.1 & 1.3.2

(Checking and Testing of Capacitor and Diode)

- Observe and follow safety policies and procedures?

- Use proper personal protective equipment?

- Did I identify different kind of electronic components?

- Follow measuring and correct ranging of Multi-meter?

- I used tools and equipments properly?

## Performance Criteria Checklist 1.3.1 & 1.3.2

(Checking and Testing of Capacitor and Diode)

| RATING | REMARKS |
| --- | --- |
| 1 | Poor |
| 2 | Fair |
| 3 | Good |
| 4 | Satisfactory |
| 5 | Excellent |