# INFORMATION SHEET No. 1.4

Transistors, Integrated Circuit's (IC's) and Transformers

## DEVICES: TRANSISTOR

It is an electronic semi-device which provides oscillation, amplification, switching and rectification of electrical current. The principal materials used are germanium and silicon. Basically, there are two kinds of transistors, namely:

Oscillation - A process of moving back and forth, to and from.

Amplification - A process of converting.

1. Positive Negative Positive (PNP)

2. Negative Positive Negative (NPN)

A transistor is an electronic amplifying device with 2 junction type. Transistors are the NPN and PNP impurity materials utilized to determine the conductivity type of a semi-conductor.

A PNP transistor is made by sandwiching a slab of silicon with "N-type" impurities between the 2 layers of silicon with "P-type" semi-conductor material. Silicon is generally used for NPN bipolar with N-type semi-conductor materials because of its lower internal voltage drop. Germanium is used for some power type transistor.

Transistors are composed of three parts – a base, a collector, and an emitter. The base is the gate controller device for the larger electrical supply. The collector is the larger electrical input, and the emitter is the outlet for that supply.

B – Base – Input

C – Collector – output

E – Emitter – Ground

### SILICON CONTROLLED RECTIFIER

A silicon-controlled rectifier (or semiconductor-controlled rectifier) is a four-layer solid state current controlling device. The name "silicon controlled rectifier" or SCR is General Electric's trade name for a type of thyristor.

SCRs are unidirectional devices (i.e. can conduct current only in one direction) as opposed to TRIACs which are bidirectional (i.e. current can flow through them in either direction). SCRs can be triggered normally only by currents going into the gate as opposed to TRIACs which can be triggered normally by either a positive or a negative current applied to its gate electrode.

## DEVICES: INTEGRATED CIRCUIT (IC)

Integrated Circuits (ICs) are used in all types of modern electronic devices. They are integrated, meaning that they are made as a total circuit and housed in one enclosure. The enclosure may take a number of shapes, it may be similar to a 2-5 transistor package with 8 leads instead of 3, it may be what is referred to as a dual in-line package (DIP) with as many as 24 leads. All components are manufactured as a common unit.

There are two major kinds of ICs:

1. Analog (or linear) which are used as amplifiers, timers and oscillators

2. Digital (or logic) which is used in microprocessors and memories

Some ICs are combinations of both analog and digital.

There are 3 categories of IC packages:

1. Small scale integration (SSI)

2. Medium scale Integration (MSI)

3. Large scale integration.

The SSI package generally has fewer than 200 components in it, MSI and LSI package may have anywhere from 1000 to 256,000 or more components.

Keep in mind that transistors, diodes, and resistors and capacitors are referred to as discrete components, and an IC may have thousands of these discrete components located on one chip. The concept of having thousand of part on one chip so small that the human eye cannot see is hard to believe when you have been working with vacuum tubes and transistor to increase this number of components on a chip.

ICs can be used to do any number of things electronically; they are classified further according to their function. The two broad categories of classification here are digital and linear.

Dual In-line Package (DIP) IC

This is an electronic device package with a rectangular housing and two parallel rows of electrical connecting pins. The package may be through-hole mounted to a printed circuit board or inserted in a socket.

Linear IC

This is a solid-state analog device characterized by a theoretically infinite number of possible operating states. It operates over a continuous range of input levels. In contrast, a digital IC has a finite number of discrete input and output states.

Ball Grid Array (BGA) IC

This is a type of surface-mount packaging used for integrated circuits. Ball-grid array (BGA) packages are used to permanently mount devices such as microprocessors.

Surface-Mounted Device (SMD) IC

This is a method for constructing electronic circuits in which the components are mounted directly onto the surface of printed circuit boards (PCBs). An electronic device so made is called a surface-mount device (SMD).

## DEVICES: TRANSFORMERS & POWERSUPPLY

### Types of Power Supply

There are many types of power supply. Most are designed to convert high voltage AC mains electricity to a suitable low voltage supply for electronic circuits and other devices. A power supply can be broken down into a series of blocks, each of which performs a particular function.

For example a full regulated supply:

Block Diagram of a Regulated Power Supply System:

220V AC Mains → Transformer → Rectifier → Smoothing → Regulator → Regulated 5V DC

Each of the blocks is described in more detail below:

Transformer - steps down high voltage AC mains to low voltage AC.

Rectifier - converts AC to DC, but the DC output is varying.

Smoothing - smooths the DC from varying greatly to a small ripple.

Regulator - eliminates ripple by setting DC output to a fixed voltage.

Power supplies made from these blocks are described below with a circuit diagram and a graph of their output:

- Transformer only

- Transformer + Rectifier

- Transformer + Rectifier + Smoothing

- Transformer + Rectifier + Smoothing + Regulator

Transformer only

Input: high voltage AC (mains supply) → Output: low voltage AC

The low voltage AC output is suitable for lamps, heaters and special AC motors. It is not suitable for electronic circuits unless they include a rectifier and a smoothing capacitor.

Transformer + Rectifier

Output: varying DC

The varying DC output is suitable for lamps, heaters and standard motors; it is not suitable for electronic circuits unless they include a smoothing capacitor.

Transformer + Rectifier + Smoothing

Output: smooth DC

The smooth DC output has a small ripple. It is suitable for most electronic circuits.

Transformer + Rectifier + Smoothing + Regulator

Output: regulated DC

The regulated DC output is very smooth with no ripple. It is suitable for all electronic circuits.

Dual Supplies

Some electronic circuits require a power supply with positive and negative outputs as well as zero volts (0V). This is called a dual supply because it is like two ordinary supplies connected together as shown in the diagram.

Dual supplies have three outputs, for example a 15V supply has +9V, 0V and -9V outputs.

### Transformer

Transformers convert AC electricity from one voltage to another with little loss of power. Transformers work only with AC and this is one of the reasons why mains electricity is AC.

Step up transformers increase voltage, step-down transformers reduce voltage. Most power supplies use a step-down transformer to reduce the dangerously high mains voltage (220V) to a safer low voltage.

3 Parts of Transformer:

Core, Primary, Secondary

The input coil is called the primary and the output coil is called the secondary. There is no electrical connection between the two coils, instead they are linked by an alternating magnetic field created in the soft iron core of the transformer. The two lines in the middle of the circuit symbol represent the core.

Transformers waste very little power so the power out is (almost) equal to the power in. Note that as voltage is stepped down current is stepped up.

The ratio of the number of turns on each coil, called the turn's ratio, determines the ratio of the voltages. A step-down transformer has a large number of turns on its primary (input) coil which is connected to the high voltage mains supply, and a small number of turns on its secondary (output) coil to give a low output voltage.

Kinds of transformer:

1. Power Transformer

2. Isolation Transformer

3. Auto Transformer

4. Audio Transformer

5. RF and IF Transformer

Types of Power Transformer:

- Center Tap Transformer

- Multi Tap Transformer

### Rectifier

There are several ways of connecting diodes to make a rectifier to convert AC to DC. A full-wave rectifier, the bridge rectifier is the most important and it produces full-wave varying DC. But this method is costly. It can also be made from just two diodes if a center-tap transformer is used. A single diode can be used as a rectifier but it only produces half-varying DC.

Bridge rectifier

A bridge rectifier can be made using four individual diodes, but it is also available in special packages containing the four diodes required. It is called a full-wave rectifier because it uses all the AC wave (both positive and negative sections). 1.4V is used up in the bridge rectifier because each diode uses 0.7V when conducting and there are always two diodes conducting, as shown in the diagram below.

Bridge rectifier: Alternate pairs of diodes conduct, changing over the connections so the alternating directions of AC are converted to the one direction of DC.

Output: full-wave varying DC (using all the AC wave)

Single diode rectifier

A single diode can be used as a rectifier but this produces half-wave varying DC which has gaps when the AC is negative. It is hard to smooth this sufficiently well to supply electronic circuits unless they require a very small current so the smoothing capacitor does not significantly discharge during the gaps.

Output: half-wave varying DC (using only half the AC wave)

### Smoothing

Smoothing is performed by a large value electrolytic capacitor connected across the DC supply to act as a reservoir, supplying current to the output when the varying DC voltage from the rectifier is falling. The diagram shows the unsmoothed varying DC (dotted line) and the smoothed DC (solid line).

The capacitor charges quickly near the peak of the varying DC, and then discharges as it supplies current to the output.

Smoothing is not perfect due to the capacitor voltage falling a little as it discharges, giving a small ripple voltage. For many circuits a ripple which is 10% of the supply voltage is satisfactory and the equation below gives the required value for the smoothing capacitor. A larger capacitor will give fewer ripples. The capacitor value must be doubled when smoothing half-wave DC.

### Regulator

Voltage regulator ICs are available with fixed (usually 5, 12 and 15V) or variable output voltages. They are also rated by the maximum current they can pass. Negative voltage regulators are available, mainly for use in dual supplies. Most regulators include some automatic protection from excessive current ('overload protection') and overheating ('thermal protection').

Many of the fixed voltage regulator ICs has 3 leads and look like power transistors, such as the 7805 +5V 1A regulator shown on the right. They include a hole for attaching a heatsink if necessary.

(Prepare for a Self check and Task Sheet, please provide a sheet of paper as answer sheet)

## Self Check No. 1.4.1

(Transistors, Integrated Circuit's (IC's) and Transformers)

Instruction: choose the letter of the correct answer.

1. It is a semi conductor device that can be used for amplification, oscillation, rectification and switching

a. Transistor
   b. Capacitor
   c. Diode
   d. none of the above

2. A type of IC with a rectangular housing and two parallel rows of electrical connecting pin

a. Linear IC
   b. Dual In-line Package (DIP) IC
   c. Surface Mounted Device (SMD) IC
   d. None of the above

3. An electro-magnetic device that is commonly used for step-down of voltage

a. Power supply
   b. Integrated circuit
   c. Transformer
   d. none of the above

4. All types of modern electronic devices. They are integrated, meaning that they are made as a total circuit and housed in one enclosure

a. Power supply
   b. Integrated circuit
   c. Transistor
   d. none of the above

5. It is a process of a transistor, that allows to move back and forth

a. Amplification
   b. Rectification
   c. Oscillation
   d. none of the above

6-7. Give at least two (2) types of integrated circuit and illustrate how it look like.

8-10. What are the three (3) parts of Transistor

11-13. What are the three (3) parts of Transformers

14-15. Give at least two (2) types of rectification process

## Task Sheet No. 1.4.1

(Checking and Testing of Transistor)

Performance Objectives:

To understand the value of capacitor, to intensify the use of Multimeter and to identify the conditions of a capacitor.

Procedures:

1. Check the value of Transistor

2. Using the suggested range in testing a terminal of Transistor in finding the base, emitter and collector

3. Use the tester in both reverse and forward bias

4. Write down the parts, description (Q1,Q2,etc.), findings, the range use and the remarks if good or defective such as leaky, shorted or open.

5. Submit to Trainer for checking

| PARTS | DESCRIPTION | FINDINGS | RANGE | REMARKS |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

## Performance Criteria 1.4.1

(Checking and Testing of Transistor)

- Observe and follow safety policies and procedures?

- Use proper personal protective equipment?

- Did I identify different kind of electronic components?

- Follow measuring and correct ranging of Multi-meter?

- I used tools and equipments properly?

## Performance Criteria Checklist 1.4.1

(Checking and Testing of Transistor)

| RATING | REMARKS |
| --- | --- |
| 1 | Poor |
| 2 | Fair |
| 3 | Good |
| 4 | Satisfactory |
| 5 | Excellent |